UNIT-1

- INTRODUCTION

1.1 Introduction

Over the past four decades the computer industry has experienced four generations of

development. The first generation used Vacuum Tubes (1940 – 1950s) to discrete diodes to transistors (1950 – 1960s), to small and medium scale integrated circuits (1960 – 1970s) and to very large scale integrated devices (1970s and beyond). Increases in device speed and reliability and reduction in hardware cost and physical size have greatly enhanced computer performance. The relationships between data, information, knowledge and intelligence are demonstrated. Parallel processing demands concurrent execution of many programs in a computer. The highest level of parallel processing is conducted among multiple jobs through multiprogramming, time sharing and multiprocessing

- EVALUATION OF COMPUTER SYSTEM

Over the past four decades the computer industry has experienced four generations of

development.

1.2.2 Generations Of Computer Systems

First Generation (1939-1954) - Vacuum Tube

 1937 - John V. Atanasoff designed the first digital electronic computer.

 1939 - Atanasoff and Clifford Berry demonstrate in Nov. the ABC prototype.

2

 1941 - Konrad Zuse in Germany developed in secret the Z3.

 1943 - In Britain, the Colossus was designed in secret at Bletchley Park to decode

German messages.

 1944 - Howard Aiken developed the Harvard Mark I mechanical computer for the Navy.

 1945 - John W. Mauchly and J. Presper Eckert built ENIAC(Electronic Numerical

Integrator and Computer) at U of PA for the U.S. Army.

 1946 - Mauchly and Eckert start Electronic Control Co., received grant from National

Bureau of Standards to build a ENIAC-type computer with magnetic tape input/output,

renamed UNIVAC( in 1947 but run out of money, formed in Dec. 1947 the new company

Eckert-Mauchly Computer Corporation (EMCC).

 1948 - Howard Aiken developed the Harvard Mark III electronic computer with 5000

tubes

 1948 - U of Manchester in Britain developed the SSEM Baby electronic computer with

CRT memory

 1949 - Mauchly and Eckert in March successfully tested the BINAC stored-program

computer for Northrop Aircraft, with mercury delay line memory and a primitive

magentic tape drive; Remington Rand bought EMCC Feb. 1950 and provided funds to

finish UNIVAC

 1950- Commander William C. Norris led Engineering Research Associates to develop the

Atlas, based on the secret code-breaking computers used by the Navy in WWII; the Atlas

was 38 feet long, 20 feet wide, and used 2700 vacuum tubes

 In 1950, the first stored program computer,EDVAC(Electronic Discrete Variable

Automatic Computer), was developed.

 1954 - The SAGE aircraft-warning system was the largest vacuum tube computer system

ever built. It began in 1954 at MIT's Lincoln Lab with funding from the Air Force. The

first of 23 Direction Centers went online in Nov. 1956, and the last in 1962. Each Center

had two 55,000-tube computers built by IBM, MIT, AND Bell Labs. The 275-ton

computers known as "Clyde" were based on Jay Forrester's Whirlwind I and had

magnetic core memory, magnetic drum and magnetic tape storage. The Centers were

connected by an early network, and pioneered development of the modem and graphics

display.

Second Generation Computers (1954 -1959) – Transistor

 1950 - National Bureau of Standards (NBS) introduced its Standards Eastern Automatic

Computer (SEAC) with 10,000 newly developed germanium diodes in its logic circuits,

and the first magnetic disk drive designed by Jacob Rabinow

 1953 - Tom Watson, Jr., led IBM to introduce the model 604 computer, its first with

transistors, that became the basis of the model 608 of 1957, the first solid-state computer

for the commercial market. Transistors were expensive at first.

 TRADIC(Transistorized digital Computer), was built by Bell Laboratories in 1954.

 1959 - General Electric Corporation delivered its Electronic Recording Machine

Accounting (ERMA) computing system to the Bank of America in California; based on a

design by SRI, the ERMA system employed Magnetic Ink Character Recognition

(MICR) as the means to capture data from the checks and introduced automation in

banking that continued with ATM machines in 1974.

3

 The first IBM scientific ,transistorized computer, IBM 1620, became available in 1960.

Third Generation Computers (1959 -1971) - IC

 1959 - Jack Kilby of Texas Instruments patented the first integrated circuit in Feb. 1959;

Kilby had made his first germanium IC in Oct. 1958; Robert Noyce at Fairchild used

planar process to make connections of components within a silicon IC in early 1959; the

first commercial product using IC was the hearing aid in Dec. 1963; General Instrument

made LSI chip (100+ components) for Hammond organs 1968.

 1964 - IBM produced SABRE, the first airline reservation tracking system for American

Airlines; IBM announced the System/360 all-purpose computer, using 8-bit character

word length (a "byte") that was pioneered in the 7030 of April 1961 that grew out of the

AF contract of Oct. 1958 following Sputnik to develop transistor computers for BMEWS.

 1968 - DEC introduced the first "mini-computer", the PDP-8, named after the mini-skirt;

DEC was founded in 1957 by Kenneth H. Olsen who came for the SAGE project at MIT

and began sales of the PDP-1 in 1960.

 1969 - Development began on ARPAnet, funded by the DOD.

 1971 - Intel produced large scale integrated (LSI) circuits that were used in the digital

delay line, the first digital audio device.

Fourth Generation (1971-1991) - microprocessor

 1971 - Gilbert Hyatt at Micro Computer Co. patented the microprocessor; Ted Hoff at

Intel in February introduced the 4-bit 4004, a VSLI of 2300 components, for the Japanese

company Busicom to create a single chip for a calculator; IBM introduced the first 8-inch

"memory disk", as it was called then, or the "floppy disk" later; Hoffmann-La Roche

patented the passive LCD display for calculators and watches; in November Intel

announced the first microcomputer, the MCS-4; Nolan Bushnell designed the first

commercial arcade video game "Computer Space"

 1972 - Intel made the 8-bit 8008 and 8080 microprocessors; Gary Kildall wrote his

Control Program/Microprocessor (CP/M) disk operating system to provide instructions

for floppy disk drives to work with the 8080 processor. He offered it to Intel, but was

turned down, so he sold it on his own, and soon CP/M was the standard operating system

for 8-bit microcomputers; Bushnell created Atari and introduced the successful "Pong"

game

 1973 - IBM developed the first true sealed hard disk drive, called the "Winchester" after

the rifle company, using two 30 Mb platters; Robert Metcalfe at Xerox PARC created

Ethernet as the basis for a local area network, and later founded 3COM

 1974 - Xerox developed the Alto workstation at PARC, with a monitor, a graphical user

interface, a mouse, and an ethernet card for networking

 1975 - the Altair personal computer is sold in kit form, and influenced Steve Jobs and

Steve Wozniak

 1976 - Jobs and Wozniak developed the Apple personal computer; Alan Shugart

introduced the 5.25-inch floppy disk

 1977 - Nintendo in Japan began to make computer games that stored the data on chips

inside a game cartridge that sold for around $40 but only cost a few dollars to

manufacture. It introduced its most popular game "Donkey Kong" in 1981, Super Mario

Bros in 1985

4

 1978 - Visicalc spreadsheet software was written by Daniel Bricklin and Bob Frankston

 1979 - Micropro released Wordstar that set the standard for word processing software

 1980 - IBM signed a contract with the Microsoft Co. of Bill Gates and Paul Allen and

Steve Ballmer to supply an operating system for IBM's new PC model. Microsoft paid

$25,000 to Seattle Computer for the rights to QDOS that became Microsoft DOS, and

Microsoft began its climb to become the dominant computer company in the world.

 1984 - Apple Computer introduced the Macintosh personal computer January 24.

 1987 - Bill Atkinson of Apple Computers created a software program called HyperCard

that was bundled free with all Macintosh computers.

Fifth Generation (1991 and Beyond)

 1991 - World-Wide Web (WWW) was developed by Tim Berners-Lee and released by

CERN.

 1993 - The first Web browser called Mosaic was created by student Marc Andreesen and

programmer Eric Bina at NCSA in the first 3 months of 1993. The beta version 0.5 of X

Mosaic for UNIX was released Jan. 23 1993 and was instant success. The PC and Mac

versions of Mosaic followed quickly in 1993. Mosaic was the first software to interpret a

new IMG tag, and to display graphics along with text. Berners-Lee objected to the IMG

tag, considered it frivolous, but image display became one of the most used features of

the Web. The Web grew fast because the infrastructure was already in place: the Internet,

desktop PC, home modems connected to online services such as AOL and CompuServe.

 1994 - Netscape Navigator 1.0 was released Dec. 1994, and was given away free, soon

gaining 75% of world browser market.

 1996 - Microsoft failed to recognize the importance of the Web, but finally released the

much improved browser Explorer 3.0 in the summer.

- TRENDS OF PARALLEL PROCESSING

From an application point of view, the mainstream of usage of computer is experiencing a

trend of four ascending levels of sophistication:

 Data processing

 Information processing

 Knowledge processing

 Intelligence processing

Computer usage started with data processing, while is still a major task of today’s computers. With more and more data structures developed, many users are shifting to computer roles from pure data processing to information processing. A high degree of parallelism has been

found at these levels. As the accumulated knowledge bases expanded rapidly in recent years,

there grew a strong demand to use computers for knowledge processing. Intelligence is very

difficult to create; its processing even more so. Todays computers are very fast and obedient and have many reliable memory cells to be qualified for data-information-knowledge processing. Computers are far from being satisfactory in performing theorem proving, logical inference and creative thinking.

5

From an operating point of view, computer systems have improved chronologically in four

phases:

 batch processing

 multiprogramming

 time sharing

 multiprocessing

In these four operating modes, the degree of parallelism increase sharply from phase to phase.

We define parallel processing as

Parallel processing is an efficient form of information processing which emphasizes the exploitation of concurrent events in the computing process. Concurrency implies parallelism,

simultaneity, and pipelining. Parallel processing demands concurrent executiom of many programs in the computer. The highest level of parallel processing is conducted among multiple jobs or programs through multiprogramming, time sharing, and multiprocessing.

Parallel processing can be challenged in four programmatic levels:

 Job or program level

 Task or procedure level

 Interinstruction level

 Intrainstruction level

The highest job level is often conducted algorithmically. The lowest intra-instruction level is often implemented directly by hardware means. Hardware roles increase from high to low levels. On the other hand, software implementations increase from low to high levels.

Information Processing

Increasing Complexity and Sophistication in Processing

Intelligence

Processing

Knowledge

Processing

Data Processing

Increasing Volumes of raw material to be processed

4 PARALLELLISM IN UNIPROCESSOR SYSTEM

1.1 Parallelism in Uniprocessor Systems

A typical uniprocessor computer consists of three major components: the main memory, the central processing unit (CPU), and the input-output (I/O) subsystem. The architectures of two commercially available uniprocessor computers are given below to show the possible interconnection of structures among the three subsystems. There are sixteen 32-bit general purpose registers, one of which serves as the program Counter (pc).there is also a special CPU status register containing information about the current state of the processor and of the program being executed. The CPU contains an arithmetic and logic unit (ALU) with an optional floating-point accelerator, and some local cache memory with an optional diagnostic memory.

1.2 Basic Uniprocessor Architecture

The CPU, the main memory and the I/O subsystems are all connected to a common bus, the

synchronous backplane interconnect (SBI) through this bus, all I/O device scan communicate

with each other, with the CPU, or with the memory. Peripheral storage or I/O devices can be

connected directly to the SBI through the unibus and its controller or through a mass bus and its controller.

The CPU contains the instruction decoding and execution units as well as a cache. Main memory is divided into four units, referred to as logical storage units that are four-way interleaved. The storage controller provides mutltiport connections between the CPU and the

four LSUs. Peripherals are connected to the system via high speed I/O channels which operate asynchronously with the CPU.

Central Processing Unit (CPU)

LOGICAL STORAGE UNITS

STORAGE CONTROLLER

LSU0 LSU1 LSU2 LSU3

I/O CHANNELS

I/O Sub System

1.3 Parallel Processing Mechanism

A number of parallel processing mechanisms have been developed in uniprocessor

computers.

We identify them in the following six categories:

 multiplicity of functional units

 parallelism and pipelining within the CPU

 overlapped CPU and I/O operations

 use of a hierarchical memory system

 multiprogramming and time sharing

 multiplicity of functional units

1.4 Multiplicity of Functional Units

The early computer has only one ALU in its CPU and hence performing a long sequence

of ALU instructions takes more amount of time. The CDC-6600 has 10 functional units built into its CPU. These 10 units are independent of each other and may operate simultaneously.

A score board is used to keep track of the availability of the functional units and registers being demanded. With 10 functional units and 24 registers available, the instruction issue rate can be significantly increased. Another good example of a multifunction uniprocessor is the IBM 360/91 which has 2 parallel execution units. One for fixed point arithmetic and the other for floating point arithmetic. Within the floating point E-unit are two functional units: one for floating point add- subtract and other for floating point multiply – divide. IBM 360/91 is a highly pipelined, multifunction scientific uniprocessor.

Parallelism And Pipelining Within The Cpu

Parallel adders, using such techniques as carry-look ahead and carry –save, are now built into almost all ALUs. This is in contrast to the bit serial adders used in the first generation machines. High speed multiplier recording and convergence division are techniques for exploring parallelism and the sharing of hardware resources for the functions of multiply and

Divide. The use of multiple functional units is a form of parallelism with the CPU. Various phases of instructions executions are now pipelined, including instruction fetch, decode, operand fetch, arithmetic logic execution, and store result.

Overlapped CPU and I/O Operations

I/O operations can be performed simultaneously with the CPU competitions by using separate I/O controllers, channels, or I/O processors. The direct memory access (DMA) channel can be used to provide direct information transfer between the I/O devices and the main memory. The DMA is conducted on a cycle stealing basis, which is apparent to the CPU.

Use of Hierarchical Memory System

The CPU is 1000 times faster than memory access. A hierarchical memory system can be used to close up the speed gap. The hierarchical order listed is

 registers

 Cache

 Main Memory

 Magnetic Disk

 Magnetic Tape

The inner most level is the register files directly addressable by ALU.

Cache memory can be used to serve as a buffer between the CPU and the main memory. Virtual memory space can be established with the use of disks and tapes at the outer levels.

Balancing Of Subsystem Bandwidth

CPU is the fastest unit in computer. The bandwidth of a system is defined as the number of operations performed per unit time. In case of main memory the memory bandwidth is measured by the number of words that can be accessed per unit time.

Bandwidth Balancing Between CPU and Memory

The speed gap between the CPU and the main memory can be closed up by using fast cache memory between them. A block of memory words is moved from the main memory into the cache so that immediate instructions can be available most of the time from the cache.

Bandwidth Balancing Between Memory and I/O Devices

Input-output channels with different speeds can be used between the slow I/O devices and the main memory. The I/O channels perform buffering and multiplexing functions to transfer the data from multiple disks into the main memory by stealing cycles from the CPU.

Multiprogramming

Within the same interval of time, there may be multiple processes active in a computer, competing for memory, I/O and CPU resources. Some computers are I/O bound and some are

CPU bound. Various types of programs are mixed up to balance bandwidths among functional units.

Example Whenever a process P1 is tied up with I/O processor for performing input output operation at the same moment CPU can be tied up with an process P2. This allows simultaneous execution of programs. The interleaving of CPU and I/O operations among several programs is called as Multiprogramming.

Time-Sharing

The mainframes of the batch era were firmly established by the late 1960s when advances in semiconductor technology made the solid-state memory and integrated circuit feasible. These

advances in hardware technology spawned the minicomputer era. They were small, fast, and

inexpensive enough to be spread throughout the company at the divisional level. Multiprogramming mainly deals with sharing of many programs by the CPU. Sometimes high priority programs may occupy the CPU for long time and other programs are put up in queue. This problem can be overcome by a concept called as Time sharing in which every process is allotted a time slice of CPU time and thereafter after its respective time slice is  over CPU is allotted to the next program if the process is not completed it will be in queue waiting for the second chance to receive the CPU time

- PARALLEL COMPUTER STRUCTURE

- PIPELINES ARRAY

- MULTIPROCESSOR

- PERFORMANCE OF PARALLEL COMPUTER DATAFLOW ARCHICTURE CLASSIFICATION APPLIOCATION