UNIT 4

MICROPROCESSOR ARCHITECHTURE AND PROGRAMMING

- FUNCTIONAL STRUCTURE

Multiproceesors are characterized by 2 attributes: First a multiprocessor is a single computer that

includes multiple processor and second processors may communicate and cooperate at different

levels in solving a given problem. The communication may occur by sending messages from one

processor to the other by sharing a common memory.

2 Different ArchitecturalModels of Multiprocessor are

Tightly Coupled System

- Tasks and/or processors communicate in a highly synchronized fashion

- Communicates through a common shared memory

- Shared memory system

Loosely Coupled System

- Tasks or processors do not communicate in a synchronized fashion

- Communicates by message passing packets

- Overhead for data exchange is high

- Distributed memory system

  - LOOSELY COUPLED MULTIPROCESSOR

 In loosely coupled Multiprocessors each processor has a set of input-output devices and a

large local memory from where instructions and data are accessed.

 Computer Module is a combination of

o Processor

o LocalMemory

o I/O Interface

 Processes which executes on different computer modules communicate by exchanging

messages through a Message Transfer System (MTS).

 The Degree of coupling is very Loose. Hence it is often referred as distributed system.

 Loosely coupled system is usually efficient when the interactions between tasks are

minimal.

It consists of a

 processor

 local memory

 Local Input-output devices

 Interface to other computer Modules

 The interface may contain a channel and arbiter switch (CAS)

 If requests from two or more different modules collide in accessing a physical segment of the

MTs, the arbiter is responsible for choosing one of the simultaneous requests according to a

given service discipline.

 It is also responsible for delaying requests until the servicing of the selected request is

completed.

 The channel within the CAS may have a high speed communication memory which is used

for buffering block transfers of messages.

 The message transfer system is a time shared bus or a shared memory system.

17.2.3 Example of Loosely Coupled Multiprocessors : The Cm* Architecture

The example of LCS is the Cm* architecture. Each computer module of the Cm* includes a local

switch called the Slocal. The Slocal intercepts and routes the processor’s requests to the memory

and the I/O devices outside of the computer module via a map bus. It also accepts references

from other computer modules to its local memory and I/O devices. The address translation uses

the four higher bits of the processors address along with the current address by the X-bit of the

LSI -11 processor status word (PSW), to access a mapping tablewhich determines whether the

reference is local or nonlocal.

A virtual address of the nonlocal reference is formed by concatenating the nonlocal address field

given by the mapping table and the source processors identification. The virtual address is

fetched by the Kmap via the map bus in response to a service request for nonlocal access. A

number of computer modules may be connected to a map bus so that they share the use of a

single KMap. The Kmap is a processor that is responsible for mapping addresses and routing

data between Slocals.

A cluster is regared as the lowest level, is made up of computer modules, a KMap and the map

bus. Clustering can enhance the cooperative ability of the processors in a cluster to operate on

shared data with low communication overhead.

The three processors in the Kmap are the Kbus, the Linc and the Pmap. The KBus is the bus

controller which arbitrates requests to the map bus. The Pmap is the mapping processor which

responds to requests from the Kbus and Linc. It also performs most of the request processing.

The Pmap communicates with the computer modules in its cluster via the map bus which is a

packet switched bus.

Three sets of queues provide interfaces between the Kbus, Linc and the Pmap. Since PMap is

much faster than the memory in the computer modules, it is multiprogrammed to handle upto

eight concurrent requests. Each of the eight partition is called a context and exists in the Pmap.

Typically, each context processes one transaction. If one context needs to wait for a message

packet to return with the reply to some request, the PMap switches to another context that is

ready to run so that some other transaction can proceed concurrently.

  - TIGHTLY COUPLED MULTIPROCESSOR

The throughput of the hierarchical loosely coupled multiprocessor may be too slow for some

applications that require fast response times. If high speed or real time processing is required the

TCS may be used.

Two typical models are discussed

In the first model, it consists of

o p processors

o l memory modules

o d input-output channels

The above units are connected through a set of three interconnection networks namely the

o processor-memory interconnection network (PMIN)

o I/O processor interconnection network (IOPIN)

o Interrupt Signal Interconnection Network (ISIN)

The PMIN is a switch which can connect every processor to every module. It has pl set of

cross points. It is a multistage network. A memory can satisfy only one processor’s request in a

given memory cycle. Hence if more than one processors attempt to access the same memory

module, a conflict occurs and it is resolved by the PMIN.

Another method to resolve the conflict is to associate a reserved storage area with each

processor and it is called as ULM unmapped local memory. It helps in reducing the traffic in

PMIN.

Since each memory references goes through the PMIN, it encounters delay in the processors

memory switch so this can be overcome by using a private cache with each processor.

A multiprocessor organization which uses a private cache with each processor is shown. This

multiprocessor organization encounters the cache coherence problem. More than one consistent

copy of data may exist in the system.

In the figure there is a module attached to each processor that directs the memory reference to

either the ULM or the private cache of that processor. This module is called the memory map

and is similar in operation to the Slocal.

17.2.5 Example of Tightly Coupled Multiprocessor : The Cyber – 170 Architecture

This configuration has 2 subsystems.

o The central processing sub system

o The peripheral processing sub system

These subsystems have to access to a common central memory (CM) through a central memory

controller, which is essentially a high speed cross bar switch. In addition to the central memory

there is an optional secondary memory called the extended core memory (ECM) which is a low

speed random access read-write memory. The ECM and CM form a 2 level memory hierarchy.

Here the CMC becomes the switching center, which performs the combined functions of

ISIN,IOPIN, and PMIN.

CM : CentralMemory

CMC : CentralMemory Controller

CPi : ith central processor

CPS : Central Processing System

PPS : peripheral processing subsystem

  - PROCESSOR CHARACTERISTIC FOR MULTIPROCESSOR

A number of desirable architectural features are described below for a processor to be effective

in multiprocessing environment.

Processor recoverability

The process and processor are 2 different entities. If the processor fails there should be another

processor to take up the routine. Reliability of the processor should be present.

Efficient Context Switching

A general purpose register is a large register file that can be used for multi-programmed

processor. For effective utilization it is necessary for the processor to support more than one

addressing domain and hence to provide a domain change or context switching operation.

PPS

CP0 CP1

CM CMC

CPS

ECM

Large Virtual and Physical address space

A processor intended to be used in the construction of a general purpose medium to large scale

multiprocessor must support a large physical address space. In addition a large virtual space is

also needed.

Effective Synchronization Primitives

The processor design must provide some implementation of invisible actions which serve as the

basis for synchronization primitives.

Interprocessor Communication mechanism

The set of processor used in multiprocessor must have an efficient means of interprocessor

mechanism.

Instruction Set

The instruction set of the processor should have adequate facilities for implementing high level

languages that permit effective concurrency at the procedural level and for efficiently

manipulating data structures.

- INTERCONNECTION NETWORK

The organization and performance of a multiple processor system are greatly influenced

by the interconnection network used to connect them.

  - TIME SHARED AND COMMON BUS

The simplest interconnection system for multiprocessors is a communication path

connecting all of the functional units. The common path is called as time shared or common bus.

The organization is least complex and easier to configure. Such an interconnection is often a

totally passive unit having no active components such as switches. Transfer operations are

completely controlled by the bus interfaces of the sending and receiving units. Since the bus is

shared, a mechanism must provide to resolve contention. The conflict resolution methods include

static or fixed priorities, FIFO queues and daisy chaining.

A unit that wishes to initiate transfer must first determine the availability of status of the

bus, and then address the destination unit to determine its availability and capability to receive

the transfer. A command is also issued to inform the destination unit what operation it is to

perform with the data being transferred, after which the data transfer is finally initiated. A

receiving unit recognizes its address placed on the bus and responds of the control signals from

the sender.

Example of Time shared bus is PDP – 11.

Figure 18.1 A single busMultiprocessor organization

M3 wishes to communicate with S5

[1] M3 sends signals (address) on the bus that causes S5 to respond

[2] M3 sends data to S5 or S5 sends data to M3 (determined by the command line)

Multiprocessor with unidirectional buses uses both the buses and not much is actually gained.

Figure 18.2 Multi-bus multiprocessor organization

This method increases the complexity. The interconnection subsystem becomes an active device.

Characteristics that affects the performance of the bus

o Number of active devices on the bus

o Bus arbitration Algorithm

o Centralization

o Data Bandwidth

o Synchronization of data transmission

o Error detection

M3 S7 M6 S5 M4

122

Various Bus arbitration Algorithms are

The static Priority Algorithm

When multiple devices concurrently request the use of bus, the device with highest

priority is granted the bus. This method is called as daisy chaining and all the services are

assigned static priorities according to their locations along a bus grant control line. The device

closest to the bus control unit will get highest priority.

Requests are made through a common line, BRQ Bus request. If the central bus control accepts

the request it passes a BGT Bus grant signal to the concerned device iff the SACK Bus busy line

is free or idle.

Figure 18.3 Static daisy chain implementation of a system bus

The Fixed Time Slice Algorithm

The available bus band widths divided into fixed time slices and then offered to various

devices in around robin fashion. If the allotted device does not use the time slice then the time

slice is wasted. This is called as fixed time slicing or time division multiplexing

Dynamic Priority Algorithm

The priorities allocated to the devices are done dynamically and thus every device gets a

chance to use the bus and does not suffer longer turn around time.

The two algorithms are

o Least Recently used (LRU)

o Rotating daisy chain (RDC)

The LRU gives the highest priority to the requesting device that has not used the bus for the

longest interval.

In a RDC scheme, no central controller exists, and the bus grant line is connected from

the last device back to the first in a closed loop. Whichever device is granted access to the bus

serves as a bus controller for the following arbitration (an arbitrary device is selected to have

initial access to the bus). Each device’s priority for a given arbitration is determined by that

device’s distance along the bus grant line from the device currently serving as a bus controller;

the latter device has the lowest priority. Hence the priorities are dynamically with each bus cycle.

Figure 18.4 Rotating daisy chain implementation of a system bus

The first come First Served Algorithm

In the FCFS, requests are simply honored in the order received. It favors no particular

processor or device on the bus. The performance measures will be degraded in case of FCFS

since the device waiting for the bus for a longer period of time just because the request is made

late.

  - CROSSBAR SWITCH AND MULTIPORT MEMORY

If the number of buses in a time shared bus is increased, a point is reached at which there is a

separate path available for each memory unit. The interconnection networking is called as a

nonblocking crossbar.

Multiport MemoryModule

- Each port serves a CPU

MemoryModule Control Logic

- Each memory module has control logic

- Resolve memory module conflicts fixed priority among CPUs

Advantages

- Multiple paths -> high transfer rate

  - MULTISTAGE NETWORK FOR MULTIPROCESSOR

In order to design multistage networks, the basic principles involved in the construction

and control of simple crossbar has to be understood.

This 2 x 2 switch has the capability of connecting the input A to either the output labelled

0 or to the output labelled 1, depending on the value of some control bit CA of the input A. If

MM MM MM MM

CPU1

CPU2

CPU3

CPU4

Memory

CA = 0 the input is connected to the upper output, and if CA = 1, the connection is made to the

lower output. Terminal B of the switches behaves similarly with a control bit CB. The 2 x 2

module also has the capability to arbitrate between conflicting requests. If both inputs A and B

require the same output terminal, then only one of them will be connected and the other will be

blocked or rejected.

It is straight forward to construct a 1 x 2n demultiplexer using the above 2 x 2 module.

This is accomplished by constructing a binary tree of these modules. The destinations are marked

in binary, If the source A is required to connect to destination (d2d1d0)2 then the root is controlled

by bit d2, the modules in the second stage are controlled by bit d1, and the modules in the third

stage are controlled by bit d0. It is clear that A can be connected o any one of the eight output

terminals. It is also obvious that B can be switched to any one of the eight outputs. The method

of constructing the 1 x 2n demultiplexer tree can be extended to build a multistage network called

a banyan tree.

Multiprocessor with Time Shared Bus

1 Lowest overall system cost for hardware and least complex

2 Very easy to physically modify the hardware system configuration by adding or removing

functional units

3 Overall system capacity limited by bus transfer rate. Failure of the bus is a catastrophic

failure.

4 Expanding the system by addition of functional units may degrade overall system

performance

5 The system efficiency attainable is the lowest of all three basic interconnection systems.

6 This organization is usually appropriate for smaller systems only.

Multiprocessor with Crossbar Switch

1 This is the most complex interconnection system. There is a potential for the highest total

transfer rate.

2 The functional units are the simplest and cheapest since the control and switching logic is in

the switch

3 Because a basic switching matrix is required to assemble any functional units into a working

configuration, this organization is usually cost effective for multiprocessors only.

4 System expansion usually improves overall performance.

5 Theoretically, expansion of the system is limited only by the size of the switch matrix,

which can often be modularly expanded within initial design or other engineering

limitations.

6 The reliability of the switch, and therefore the system can be improved by segmentation and

/ or redundancy within the switch.

Multiprocessors with Multiport Memory

1 Requires the most expensive memory units since most of the control and switching circuitry

is included in the memory unit

2 The characteristics of the functional units permit a relatively low cost uniprocessor to be

assembled from them.

3 There is potential for a very high total transfer rate in the overall system.

4 The size and configuration options possible are determined by the number and type of

memory ports available; this design decision is made quite early in the overall design and is

difficult to modify.

5 A large number of cables and connectors are required.

  - PERFORMANCE FOR INTERCONNECTION NETWORK

- PARALLEL MEMORY ORGANISATION

  - INTERLEAVED MEMORY CONFIGURATION

  - PERFORMANCE TRADE OFFS IN MEMORY ORGANISATION

  - MULTICATCH PROBLEM AND SOLUTION