MICROPROCESSORS AND ASSEMBLY LANGUAGES

A MICROPROCESSOR is a multipurpose programmable logic device that reads binary instructions from a storage device called memory accepts binary data as input and processes data according to those

instructions and provides results.

- This chip was given the name, processor chip. The large processors were developed using VLSI technology.

- Engineers was developing a processor which worked in a con trolled manner, was given the name of microprocessor.

- Thus a single LSI/ VLSI/VVLSI chip, capable of processing data in a controlled manner was called the microprocessor.

First generation microprocessor

- The design team headed by Ted Hoff of Intel Corporation developed the 1st controlled processor in the year 1969,

- but Intel started marketing its first microprocessor in the name of Intel 4004 in 1971.

- This was a 4-bit microprocessor having 16-pins.

- The 4-bit microprocessor worked with 4-bit word .

- The Intel 8008 was developed in the year 1972 that worked with 8-bit word.

- It required about 20 or more additional devices(chips) to design a functional CPU.

Second generation microprocessors

- Large chip size (170x200 mil) with 40-pins.

- More chips on decoding circuits.

- Ability to address large memory space (64-K Byte) and I/O ports(256).

- More powerful instruction sets.

- Dissipate less power.

- Better interrupt handling facilities.

- Cycle time reduced to half (1.3 to 9 sec.)

- Sized 70x200 mil) with 40-pins.

- Less Support Chips Required

- Used Single Power Supply Faster Operation

3rd generation microprocessor

- It have 64-pins started with the introduction of 16-bit Intel

8086 in the year 1978.

- The other important IIIrd generation microprocessors were Zilog Z-8000, Motorola M68000, National NS16016, and Texas Instruments TMS 99000 series, etc.

- In addition to enhanced performance, it contained multiply/divide arithmetic hardware.

- Intel 8088 was identical to 8086 but for the 8-bit data bus.

- Hence 8088 could read or write 8-bits data at a time to or from the memory.

- In addition to 16-bit CPU, the 80186 and 80188 had programmable peripheral devices integrated on the same package.

- The program written for 80186 and 80188 may not work well on 8086 and 8088, but those written for 8086 and 8088 worked without much difficulties on 80186 and 80188.

- This means they were upward compatible with 8086 and 8088. The Intel 80286 was the advancedversionof80186.

- The single chip 32-bit microprocessor was introduced in the year 1981 by Intel as iAPX 432.

4th generation microprocessors

- Bell Single Chip Bellmac-32, Hewlett-Packard, National NSl 6032, Texas Instrument99000. Motorola 68020 and 68030.

- The power of the microprocessor went on increasing with the advancement in the integrated circuit technology.

- The VLSI technology culminated in the extremely complex microprocessor with as many as one billion transistors on a single chip.

- The Intel in the year 1985 announced the 32-bit microprocessor (80386).

- The 80486 has already been announced and is also a 32-bit microprocessor

HISTORY OF MICRO PROCESSOR

- Fairchild Semiconductors (founded in 1957)

- invented the first IC in 1959.

- In 1968, Robert Noyce, Gordan Moore, Andrew

Grove resigned from Fairchild Semiconductors.

- They founded their own company Intel

(Integrated Electronics).

- Intel grown from 3 man start-up in 1968 to industrial

giant by 1981.

- It had 20,000 employees and $188 million revenue.

INTEL 4004

- Introduced in 1971.

- It was the first microprocessor by Intel.

- It was a 4-bit μP.

- Its clock speed was 740KHz.

- It had 2,300 transistors.

- It could execute around60,000 instructions per second.

INTEL 8008

- Introduced in 1972.

- It was first 8-bit μP.

- Its clock speed was 500 KHz.

- Could execute 50,000 instructions per second.

8

INTEL 8080

- Introduced in 1974.

- It was also 8-bit μP.

- Its clock speed was 2MHz.

- It had 6,000 transistors.

- Was 10 times faster than 8008.

- Could execute 5,00,000 instructions per second.

INTEL 8085

- Introduced in 1976.

- It was also 8-bit μP.

- Its clock speed was 3 MHz.

- Its data bus is 8-bit and address bus is 16-bit.

- It had 6,500 transistors.

- Could execute 7,69,230 instructions per second.

- It could access 64 KB of memory.

- It had 246 instructions.

- Over 100 million copies were sold.

INTEL 8086

- Introduced in 1978.

- It was first 16-bit μP.

- Its clock speed is 4.77 MHz, 8 MHz and 10 MHz, depending on the version.

- Its data bus is 16-bit and address bus is 20-bit.

- It had 29,000 transistors.

- Could execute 2.5 million instructions per second.

- It could access 1 MB of memory.

- It had 22,000 instructions.

- It had Multiply and Divideinstructions.

INTEL 8088

- Introduced in 1979.

- It was also 16-bit μP.

- It was created as a cheaper version of Intel’s 8086.

- It was a 16-bit processor with an 8-bit external bus.

- Could execute 2.5 million instructions per second.

- This chip became the most popular in the computer

- industry when IBM used it for its first PC.

INTEL 80186 & 80188

- Introduced in 1982.

- They were 16-bit μPs.

- Clock speed was 6 MHz.

- 80188 was a cheaper version of 80186 with an 8-

- bit external data bus.

- They had additional components like:

- Interrupt Controller

- Clock Generator

- Local Bus Controller

- Counters

INTEL 80286

- Introduced in 1982.

- It was 16-bit μP.

- Its clock speed was 8 MHz.

- Its data bus is 16-bit and address bus is 24-bit.

- It could address 16 MB of memory.

- It had 1,34,000 transistors.

- It could execute 4 million instructions per second.

INTEL 80386

- Introduced in 1986.

- It was first 32-bit μP.

- Its data bus is 32-bit and address bus is 32-bit.

- It could address 4 GB of memory.

- It had 2,75,000 transistors.

- Its clock speed varied from 16

MHz to 33 MHz depending uponthe various versions.

- Different versions:

- 80386 DX

- 80386 SX

- 80386 SL

- Intel 80386 became the bestselling microprocessor in history.

INTEL 80486

- Introduced in 1989.

- It was also 32-bit μP.

- It had 1.2 million transistors.

- Its clock speed varied from 16 MHz to 100 MHz depending

- upon the various versions.

- It had five different versions:

- 80486 DX

- 80486 SX

- 80486 DX2

- 80486 SL

- 80486 DX4

- 8 KB of cache memory was introduced.

INTEL PENTIUM

- Introduced in 1993.

- It was also 32-bit μP.

- It was originally named 80586.

- Its clock speed was 66 MHz.

- Its data bus is 32-bit andaddress bus is 32-bit.

- It could address 4 GB of memory.

- Could execute 110 million instructions per second.

- Cache memory:

- 8 KB for instructions.

- 8 KB for data.

INTEL PENTIUM PRO

- Introduced in 1995.

- It was also 32-bit μP.

- It had L2 cache of 256 KB.

- It had 21 million transistors.

- It was primarily used in server systems.

- Cache memory:

- 8 KB for instructions.

- 8 KB for data.

- It had L2 cache of 256 KB

INTEL PENTIUM II

- Introduced in 1997.

- It was also 32-bit μP.

- Its clock speed was 233 MHz to 500 MHz.

- Could execute 333 million instructions per second.

- MMX technology was supported.

- L2 cache & processor were on one circuit

INTEL PENTIUM II XEON

- Introduced in 1998.

- It was also 32-bit μP.

- It was designed for servers.

- Its clock speed was 400 MHz to 450 MHz.

- L1 cache of 32 KB & L2 cache of 512 KB, 1MB or 2MB.

- It could work with 4 Xeons in same system.

INTEL PENTIUM III

- Introduced in 1999.

- It was also 32-bit μP.

- Its clock speed varied from 500 MHz to 1.4GHz.

- It had 9.5 million transistors.

23

INTEL PENTIUM IV

- Introduced in 2000.

- It was also 32-bit μP.

- Its clock speed was from 1.3 GHz to 3.8 GHz.

- L1 cache was of 32 KB & L2 cache of 256 KB.

- It had 42 million transistors.

- All internal connections were made from aluminiumto copper.

INTEL DUAL CORE

- Introduced in 2006.

- It is 32-bit or 64-bit μP.

- It has two cores.

- Both the cores have there own internal bus and L1 cache, but share the external bus and L2 cache (Next Slide).

- It supported SMT technology.

- SMT: Simultaneously Multi-Threading

- E.g.: Adobe Photoshop supported SMT.

INTEL CORE 2

- Introduced in 2006.

- It is a 64-bit μP.

- Its clock speed is from 1.2 GHz to 3 GHz.

- It has 291 million transistors.

- It has 64 KB of L1 cache per core and 4 MB of L2 cache.

- It is launched in three different versions:

- Intel Core 2 Duo

- Intel Core 2 Quad

- Intel Core 2 Extreme

INTEL CORE I7

- Introduced in 2008.

- It is a 64-bit μP.

- It has 4 physical cores.

- Its clock speed is from 2.66 GHz to 3.33 GHz.

- It has 781 million transistors.

- It has 64 KB of L1 cache per core, 256 KB of L2 cache and 8 MB of L3 cache

INTEL CORE I5

- Introduced in 2009.

- It is a 64-bit μP.

- It has 4 physical cores.

- Its clock speed is from 2.40 GHz to 3.60 GHz.

- It has 781 million transistors.

- It has 64 KB of L1 cache per core, 256 KB of L2 cache and 8 MB of L3 cache.

INTEL CORE I3

- Introduced in 2010.

- It is a 64-bit μP.

- It has 2 physical cores.

- Its clock speed is from 2.93 GHz to 3.33 GHz.

- It has 781 million transistors.

- It has 64 KB of L1 cache per core, 512 KB of L2 cache and 4 MB of L3 cache.

MICRO PROCESSOR ARCHITECTURE AND ITS OPERATIONS

ACCUMULATOR

TEMP. REG

FLAG FLIP FLOP

ALU

TIMING AND CONTROL

INSTRUCTION

INSTRUCTION

DECODER

AND

MACHINE CYCLE

ENCODING

MULTIPLEXER

STOCK POINTER

PROGRAM COUNTER

INCREMENT/

DECREMENT ADDRESS LATCH

ADDERS BUFFER

DATA/ADDRESS

| W | Z |
| --- | --- |
| B | C |
| D | E |
| H | L |

The Micro processor contains

- Arithmatic andLogical Unit

- General Purpose Registers

- Special Purpose Registers

- Instruction Registers

- Time and control units.

Arithmatic andLogical Unit(ALU)

- It performs the Arithmetic Operations

add, sub, inr/dcr

- It performs the Logical operation

and, or, xor, rotate, compare, complement

- It performs the Branch operation

Jump, call, return

- It performs the Data transfer,Copy, and Memory operation.

MOV, MVI, LD, ST, OUT

General Purpose Registers

- Six general purpose 8‐bit registers: B, C, D, E, H,L

- Combined as register pairs to perform 16‐bit operations: BC, DE, HL

- Registers are programmable (load, move, etc.)

Special Purpose Registers

- Accumulator

- Single 8-bit register that is part of the ALU !

- Used for arithmetic and logic operations , the result is  always stored in the accumulator.

- Flag Register

- There is also the flags register whose bits are affected by the arithmetic & logic operations.

- sign flag: The sign flag is set if bit D7 of the accumulator is set after an arithmetic or logic operation.

- zero flag : Set if the result of the ALU operation is 0. Otherwise is reset. This flag is affected by operations on the accumulator as well as other registers. (DCR B).

- AC-Auxiliary Carry: This flag is set when a carry is generated from bit D3 and passed to D4 . This flag is used only internally for BCD operations. (Section 10.5 describes BCD addition including the DAA instruction).

- Parity flag : After an ALU operation if the result has an even # of 1’s the p-flag is set. Otherwise it is cleared. So, the flag can be used to indicate even parity.

- Carry flag: Discussed earlier

- The Program Counter (PC)

- This is a register that is used to control the sequencing of the execution of instructions.

- This register always holds the address of the next instruction.

- Since it holds an address, it must be 16 bits wide.

- The Stack pointer

- The stack pointer is also a 16-bit register that is used to point into memory.

- The memory this register points to is a special area called the stack.

- The stack is an area of memory used to hold data that will be retreived soon.

- The stack is usually accessed in a Last In First Out (LIFO) fashion.

Instruction Registers

- It is 8 bit register

- It is used to temperarly stored data.

- The temperarly registers are W,Z.

Time and control units

- Data Bus

- Address Bus

- Control Bus

#### Operations of Microprocessor

- Although the 8085 is an 8-bit processor, it also has some 16-bit operations.

- Any of the three 16-bit register pairs (BC, DE, HL) or SP could be loaded with an immediate 16-bit value (using LXI), incremented or decremented (using INX and DCX), or added to HL (using DAD).

- LHLD loaded HL from directly-addressed memory and SHLD stored HL likewise.

- The XCHG operation exchanges the values of HL and DE.

- Adding HL to itself performs a 16-bit arithmetical left shift with one instruction.

- The only 16 bit instruction that affects any flag was DAD (adding HL to BC, DE, HL or SP), which updates the carry flag to facilitate 24-bit or larger additions and left shifts (for a  for instance).

- Adding the stack pointer to HL is useful for indexing variables in (recursive) stack frames.

- A stack frame can be allocated using DAD SP and SPHL, and a branch to a computed pointer can be done with PCHL.

- These abilities make it feasible to compile languages such as  with 16-bit variables and produce 8085 machine code.

- Subtraction and bitwise logical operations on 16 bits is done in 8-bit steps.

- Operations that have to be implemented by program code (subroutine libraries) included comparisons of signed integers as well as multiply and divide.

-----------------------------------Success----------------------------------

See for me;

Study for you

Save for Relation;

Stuggle for theNation

-by Puradchi R Bharathidasan