1 BOOLEAN FUNCTION

- It may be defined with a set of elements, a set of operators, and a number of unproved axioms.

- A is a set of elements which is any collection of objects having a common property.

- It contains the value of 1, 2, 3, and 4.

A = {1,2,3,4,...}

- The binary operators defined on the set S.

- S is called set which contains values x, y, and z.

- X, y € s denotes that x is a members.

- Z € s denotes y is not an element

S = {x,y,z}

- The common axioms used to various algebric structure are;

i. Closure:

- A is a set which closed with respect to a binary operator. N = (1, 2, 3, 4…)

- The set of natural numbers is not closed with respect to the binary operator (+)

- a+b = c

a,b€N

c€N

- 2-3 -1

2, 3 € N

-1 € N

ii. Assosiative law:

- S is a set with respect to a binary operator (*)

(x*y ) *z  = (x*y)*z

For all x, y, z€S

iii.Commutative law:

- S is a set with respect to a binary operators (*)

X*y = y*x

For all x, y € S

iv. Identity law:

- S is a set of which said to be an inverse with respect to a binary operator (*)

- e is an identity  element e * x = x* e =x

X€S

v. Inverse law:

- S is a set of which said to be an inverse with respect to a binary operator (*)

- e is an identity  element.

X*y = e

X€S

Y€S.

vi. Distributive law:

- S is a set of which said to be an inverse with respect to a binary operator (*and .)

X*(y.z)=(x*y).(x*z)

a.(b+c)=(a.b)+(a.c)

2. AXIOMATIC DEFINITION:

- George boole developed boolean algebra in 1854.

- C.E.Shannon introduced a two valued boolean algebra called switching algebra in 1938.

- E.V  Huntington have definition of formulated the axioms in 1904.

- Axioms are not unique for defining boolean algebra.

- Algebric structure defined 2 binary operators (+) (*).

- axioms are satisfy by using binary operators  are follow;

- Closure:

- respect to the operator:+

- respect to the operator:*

- Iidentity:

- respect to the designated by 0:

x+0=0+x=x

- respect to the operators designated by 1:

x*1=1*x=x

- Commutative :

- respect to the +: x+y=y+x

- respect to the *:x*y =y*x

- Distributive:

- over +,* : x*(y+z)=(x*y) + (x*z)

- over *,+ : x+(y*z) = (x+y)*(x+z)

- Element x€b , complement x’ € b

- x+x’= 1

- x*x’ = 0

- Two elements x,y€b such that x ≠ y

- Defferent between Normal algebra and Boolean algibra.

| sn | algebra | boolean algebra |
| --- | --- | --- |
| 1 | supports all arithmatic operation | not supports all arithmatic operation |
| 2 | not supports complement | supports complement |
| 3 | deals real numbers | deals two valued boolean algebra |
| 4 | not supports distributive law | supports distributive law |

Two-valued Boolean algebra

4. BOOLEAN FUNCTIONS:

- A binary variable can take the value of 0 and 1.

- A Boolean function is an expression  formed with.

- Binary variables (x, y)

- Binary operator (AND, OR) and

- Unary operator (NOT).

- The example for Boolean function;

F1=xyz1

x

y                                         F1

z

- The example for Boolean function;

F2=x+y1z

x

y				F2

z

- The example for Boolean function;

F3=xy’+x’y

x

y

F3

z

- The example for Boolean function;

F4=x’y+y’z+z’x

x

y

z				     F4

6. LOGICAL OPERATIONS:

Not/ complement operation:

- Not is a complement function which used the operator  (x1) Not x.

F1=x1

| x | F1 |
| --- | --- |
| 0 1 | 1 0 |

AND operation

- These function is formed by two binary variables

- It gives the value of product of function

- It used the(.) dot operator.

F2=x .y

| X | Y | F2 |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

OR operation;

- It used the operator symbol (+).

F3=x +y

| X | Y | F3 |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

Transfer operation;

- These operation is used to transfer value

- It has no operator.

F4=x

| X | F4 |
| --- | --- |
| 0 | 0 |
| 1 | 1 |

NAND operations;

- It is a complement functions of AND operation

- It is not-AND

- The operator symbols of NAND is

F5=x  y

=(x .y)1

| X | Y | F5 |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

NOR operation

- It is not-OR

- It is a complement functions of OR operation

- The operator symbols of NOR is

F6= x   y

= (x +Y)1

| X | Y | X+Y | (X+Y)’ |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 |

XOR operation

- Exclusive-OR abreacted XOR,EOR.

- The functions are the complement of each other.

- It used operator’s symbol (       ).

F7=x        y

XOR= XY’+X’Y

| X | Y | X’ | Y’ | XY’ | X’Y | XY’+X’Y |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 0 | 1 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 |

7. DIGITAL LOGIC GATES

NOT gates

- Not is a complement function

| x | F1 |
| --- | --- |
| 0 | 1 |
| 1 | 0 |

F1=x1

x                        F1=x1

Buffer/transfer gate:

- Buffer gate is used to transfer the value of function

F2=x

| x | F2 |
| --- | --- |
| 0 | 0 |
| 1 | 1 |

x                        F2=x

AND gate

- These function is formed by two binary variables

- It gives the value of product of function

- It used the(.) dot operator.

| X | Y | F2 |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

F2=x .y

OR gate;

- It used the operator symbol (+).

F3=x +y

| X | Y | F3 |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

NAND gate;

- It is a complement functions of AND operation

- It is not-AND

- The operator symbols of NAND is

F5=x  y

=(x .y)1

| X | Y | F5 |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

NOR gate

- It is not-OR

- It is a complement functions of OR operation

- The operator symbols of NOR is

F6= x   y

= (x +Y)1

| X | Y | X+Y | (X+Y)’ |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 |

XOR operation

- Exclusive-OR abreacted XOR,EOR.

- The functions are the complement of each other.

- It used operator’s symbol (       ).

F7=x        y

XOR= XY’+X’Y

| X | Y | X’ | Y’ | XY’ | X’Y | XY’+X’Y |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 0 | 1 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 |

5. CONANICAL AND STANDARD FORM

MINTERM:

- A binary variable appear in normal form(x) in its compliment form (x’).

- It may appear in either form there are four possibilities.

- x’y’, x’y, xy’, xy

- Four AND term represent one of the distinct areas is called minterm.

MAX TERM:

- Four possible combination of OR binary values x’+y’, x’+y, x+y’, x+y.

- These four term of distinct area is called maximum term.

- The procedures for obtaining product of max term directly form the truth table.

- From a max term for each combination of the variable which produces a 0 in the function AND of all those max term.

- Boolean function expressed as a some of min term of product max term as set to be a canonical form.

Sum of Minterms

The Boolean function F=A+B’C

A=A(B+B’)

=AB+AB’

=AB(C+C’)=AB’(C+C’)

=ABC+ABC’+AB’C+AB’C’

B’C= B’C(A+A’)

=AB’C+A’B’C

A+B’C   = ABC+ABC’+AB’C+AB’C’+AB’C+A’B’C

=m1+M4+m5+m6+m7

F(A,B,C)=£(1,4,5,6,7)

Product of Max terms

The Boolean function F=xy+X’z

=xy+x’z

=(xy+x’)(xy+z)

=(x+x’)(y+x’)(x+z)(y+z)

= (y+x’)(x+z)(y+z)

(x’+y)= (x’+y+z) (x’+y+z’)

(x+z)= (x+y+z) (x+y’+z)

(y+z)= (x+y+z) (x’+y+z)

F=(x’+y+z) (x’+y+z’) (x+y+z) (x+y’+z) (x+y+z) (x’+y+z)

F=(x’+y+z) (x’+y+z’) (x+y+z) (x+y’+z)

=M0.M2.M4.M5

F(x,y,z)=π(0,2,4,5)

Canonical Form

Sum of minterms            F(A,B,C)=£(1,4,5,6,7)

Compliment of F	F(A,B,C)’=£(0,2,3)

Product of max terms  F(x,y,z)= π(0,2,3)

The De Morgon’s Theorem

F(A,B,C)’=(m0+m2+m3)

F(x,y,z)=π(M0.M2.M3)

M’j=Mj